Department of Electronics and Computers

Study programme: Applied Electronics

Verification Project

Environmental Control Device

Table of Contents

1	The digital design of the circuit	4

1.1	Block Diagram	4

1.2	Description of the general functionality of the DUT (Device Under Test)	4

1.3	DUT Interfaces	5

1.4	List of DUT Functionalities	6

1.5	Waveforms	7

2	DIGITAL CIRCUIT VERIFICATION	8

2.1	Verification Environment Architecture	8

2.1.1	Sensor Agent	8

2.1.2	Sensor Transaction	9

2.1.3	Sensor Driver	9

2.1.4	Sensor Monitor	9

2.1.5	Sensor Coverage Collector	9

2.1.6	Sensor Sequences	9

2.1.7	Actuator Agent	10

2.1.8	Actuator Transaction	10

2.1.9	Actuator Monitor	10

2.1.10	Actuator Coverage Collector	10

2.1.11	Button Agent	10

2.1.12	Button Transaction	11

2.1.13	Button Driver	11

2.1.14	Button Monitor	11

2.1.15	Button Coverage Collector	11

2.1.16	Button Sequences	11

2.1.17	Scoreboard	11

2.1.18	Scoreboard Coverage Collector	12

2.2	Verification Checklist	12

2.2.1	Assertions	12

2.3	Functional Coverage Elements	13

2.4	Verification Scenarios and Tests	15

2.5	Results	16

# The digital design of the circuit

Block Diagram General Functionality Description of the DUT (Device Under Test) DUT Interfaces List of DUT Functionalities Waveforms

## Block Diagram

Figure 1. Block diagram of the device

## Description of the general functionality of the DUT (Device Under Test)

The environmental controller receives temperature, humidity and light intensity measurements from sensors and controls heating, air conditioning, a dehumidifier and automatic blinds.

The button behaves as a toggle switch. When pressed, Enable becomes 1 and the controller starts operating. When pressed again, Enable becomes 0 and the controller is disabled.

Table 1. Sensor ranges

| Sensor measurement | Min Value | Max Value |
| --- | --- | --- |
| Temperature | 0 | 40 |
| Humidity | 0 | 100 |
| Light Intensity | 0 | 900 |

Based on the temperature sensor, the heating system turns on when the temperature drops below 22 degrees Celsius. The air conditioner turns on when the temperature exceeds 25 degrees Celsius and turns off when it reaches 22 degrees.

The values received from the humidity sensor control the dehumidifier. It turns on at a value exceeding 50% and turns off at values between 0% and 35%.

Depending on the light intensity levels, the curtains are opened or closed to limit the amount of light. The recommended light level in an office is 300–500 lux; thus, when the threshold of 700 lux is exceeded, the curtain closing system will activate, and at 200 lux, the curtains will open.

If the button is not pressed (enable=0) or the reset is active (reset_n = 1), all DUT outputs are set to 0. After the button is pressed again (enable = 1), the DUT outputs will be updated only after new data is read from the sensors.

Data received when the module’s enable signal is 0 is ignored (OFF state). Data is read only when enable, valid and ready signals are all asserted (load state). In the control state, various commands are executed based on the information received from the sensors.

Figure 2. Finite State Machine describing the functionality of the device

## DUT Interfaces

Table 2. System signals generated in the testbench

| General Signals |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| clk_i | I | 1 | Clock signal |
| reset_i | I | 1 | Asynchronous reset, active high |

Table 3. Button Interface

| Button Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| enable_i | I | 1 | For enabling/disabling device |

Table 4. Sensor Interface

| Sensor Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| temperature_i | I | 6 | Temperature data |
| humidity_i | I | 7 | Humidity data |
| luminous_intensity_i | I | 10 | Luminous intensity data |
| valid_i | I | 1 | Valid signal |
| ready_o | O | 1 | Ready signal |

Table 5. Actuator Interface

| Actuator Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| heat_o | O | 1 | Heating status |
| AC_o | O | 1 | AC status |
| dehumidifier_o | O | 1 | Dehumidifier status |
| blinds_o | 0 | 1 | Blinds status |

## List of DUT Functionalities

- Reads button state for enabling/disabling

- Receives environmental data from sensors

- Processes sensor data

- Enables/disables heat, AC, dehumidifier and blinds

## Waveforms

Figure 3. Waveforms from the DUT

# DIGITAL CIRCUIT VERIFICATION

Verification Environment Architecture Verification Checklist Functional Coverage Elements Verification Scenarios Tests Results

## Verification Environment Architecture

Figure 4. Overview of the components necessary for sending stimuli to the DUT

### Sensor Agent

The Sensor Agent is an active agent that encapsulates the Sequencer, Driver and Monitor into a single entity, connecting the components with the DUT using the virtual interface.

### Sensor Transaction

Table 6. Sensor Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| temperature | rand bit | 6 | [0:40] | Value of environmental temperature |
| humidity | rand bit | 7 | [0:100] | Value of environmental humidity |
| luminous_intensity | rand bit | 10 | [0:900] | Value of environmental light intensity |

### Sensor Driver

The Sensor Driver interacts with the DUT via the Sensor Interface, sending the transactions generated by the Sensor Sequencer. The transactions follow the valid-ready protocol.

The driver receives transactions from the sequencer, waits for the ready signal to assert, asserts valid signal and drives sensors values, and after a clock cycle de-asserts the valid signal.

### Sensor Monitor

The Sensor Monitor waits for valid and ready signals to be both asserted, samples the sensors values from the Sensor Virtual Interface, reconstructs the transaction and sends it to the Scoreboard and Sensor Coverage Collector.

### Sensor Coverage Collector

The Sensor Coverage Collector is instantiated inside the Sensor Monitor and receives transactions from it. The coverage collector contains sensor_cg cover group with 4 cover points: temperature_cp, ac_cp, humidity_cp, luminous_intensity_cp.

### Sensor Sequences

The Sensor Sequences generate stimulus for the Sensor Driver. Each sequence creates one or more constrained-random transactions that the driver sends to Sensor Interface. The following sequences are implemented:

Table 7. Sensor Sequences

| Sequence | Description |
| --- | --- |
| temperature_sequence | Sends values of environmental temperature for every state of heat and AC. |
| humidity_sequence | Sends values of environmental humidity for every state of dehumidifier. |
| luminosity_sequence | Sends values of environmental light intensity for every state of blinds. |
| sensor_limit_values_sequence | Sends only lower and upper limits of every environmental data range. |
| random_sensor_sequence | Sends random values of environmental data. |

### Actuator Agent

The Actuator Agent is a passive agent that contains only the Monitor and connects it with the DUT using the virtual interface. The Actuator Interface has only output signals from DUT, meaning no driver or sequencer is needed.

### Actuator Transaction

Table 8. Actuator Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| Heat_i | rand bit | 1 | [0:1] | Heat command |
| AC_i | rand bit | 1 | [0:1] | AC command |
| Blinds_i | rand bit | 1 | [0:1] | Blinds command |
| Dehumidifier_i | rand bit | 1 | [0:1] | Dehumidifier command |

### Actuator Monitor

The Actuator Monitor samples the commands sent by DUT every time each of the values changes, reconstructs the transaction and sends it to the Scoreboard and Actuator Coverage Collector.

### Actuator Coverage Collector

The Actuator Coverage Collector is instantiated inside the Actuator Monitor and receives transactions from it. The coverage collector contains actuator_cg cover group with 4 coverpoints: heat, ac, blinds, dehumidifier; and actuator_cross cross.

### Button Agent

The Button Agent is an active agent that encapsulates the Sequencer, Driver and Monitor into a single entity, connecting the components with the DUT using the virtual interface.

### Button Transaction

Table 9. Button Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| enable | rand bit | 1 | [0:1] | Value of enable signal |

### Button Driver

The Button Driver receives transactions from the Button Sequencer and drives the enable signal.

### Button Monitor

The Button Monitor waits for the enable signal to change, reconstructs the transaction and sends it to the Scoreboard and Button Coverage Collector.

### Button Coverage Collector

The Button Coverage Collector is instantiated inside the Button Monitor and receives transactions from it. The coverage collector contains button_cg cover group with button_state cover point.

### Button Sequences

The Button Sequences generate stimulus for the Button Driver. Each sequence creates one or more constrained-random transactions that the driver sends to Button Interface. The following sequences are implemented:

Table 10. Button Sequences

| Sequence | Description |
| --- | --- |
| fast_switch_sequence | Alternative values for enable signal. |
| frequent_button_sequence | Device is mostly enabled. |
| rare_button_sequence | Device is mostly disabled. |
| button_sequence | Random values for enable signal. |

### Scoreboard

The Scoreboard receives transactions from all monitors.

When receiving a Button Transaction, the scoreboard stores the value of enable signal and predicts an implicit transaction from Sensor Monitor if enable is 0.

When receiving a Sensor Transaction, predicts the next Actuator Transaction if the stored enable signal value is 1.

All predicted Actuator Transactions are stored in a queue.

When receiving an Actuator Transaction, the scoreboard compares it with the predicted transaction, generating an error message if the checker fails.

### Scoreboard Coverage Collector

The Scoreboard Coverage Collector is instantiated inside the Scoreboard and receives transactions from it. The coverage collector contains processed_data_cg cover group with 4 cover points: scbd_enable_cp, scbd_temperature_cp, scbd_humidity_cp, scbd_luminous_intensity_cp; and 3 crosses: temperature_cross, humidity_cross, luminous_intensity_cross.

## Verification Checklist

### Assertions

All Sensor assertions are disabled when reset is active to prevent false assertion failures.

Table 11. Actuator Interface Assertions

| Assertion | Description |
| --- | --- |
| temperature_control | Verifies that heat and AC are not on at the same time. |

Table 12. Sensor Interface Assertions

| Assertion | Description |
| --- | --- |
| temperature_range | Verifies that temperature signal is in the interval [0,40]. |
| humidity_range | Verifies that humidity signal is in the interval [0,100]. |
| luminous_intensity_range | Verifies that luminous intensity signal is in the interval [0,900]. |

## Functional Coverage Elements

Table 13. Actuator cover group

| actuator_cg |  |  |
| --- | --- | --- |
| Cover point | Values | Description |
| heat | [0:1] | Covers heat command |
| ac | [0:1] | Covers AC command |
| blinds | [0:1] | Covers blinds command |
| dehumidifier | [0:1] | Covers dehumidifier command |
| actuator_cross | heat x ac x blinds x dehumidifier | Covers all combinations of commands |

Table 14. Sensor cover group

| button_cg |  |  |
| --- | --- | --- |
| Cover point | Values | Description |
| button_state | [0:1] | Covers device state |

Table 15. Sensor cover group

| sensor_cg |  |  |  |
| --- | --- | --- | --- |
| Cover point | Bins | Values | Description |
| temperature_cp | heat_on | [0:21] | Temperature for heat on |
|  | heat_limit_on | 22 | Temperature limit value for heat on |
|  | heat_limit_off | 23 | Temperature limit value for heat off |
|  | heat_off | [24:40] | Temperature for heat off |
| ac_cp | ac_off | [0:21] | Temperature for ac off |
|  | ac_limit_off | 22 | Temperature limit value for ac off |
|  | ac_stable | [23:24] | Temperature for ac stable |
|  | ac_limit_on | 25 | Temperature limit value for ac on |
|  | ac_on | [26:40] | Temperature for ac on |
| humidity_cp | dehumidifier_off | [0:34] | Humidity for dehumidifier off |
|  | dehumidifier_limit_off | 35 | Humidity limit value for dehumidifier off |
|  | dehumidifier_stable | [36:49] | Humidity value for dehumidifier stable |
|  | dehumidifier_limit_on | 50 | Humidity limit value for dehumidifier on |
|  | dehumidifier_on | [51:100] | Humidity for dehumidifier on |
| luminous _intensity_cp | blinds_open | [0:199] | Luminous intensity for blinds open |
|  | blinds_limit_open | 200 | Luminous intensity limit value for blinds open |
|  | blins_stable | [201:699] | Luminous intensity for blinds stable |
|  | blinds_limit_close | 700 | Luminous intensity limit value for blinds closed |
|  | blinds_closed | [701:900] | Luminous intensity for blinds closed |

Table 16. Scoreboard cover group

| processed_data_cg |  |  |
| --- | --- | --- |
| Cover point | Values | Description |
| scbd_enable_cp | [0:1] | Covers device state |
| scbd_temperature_cp | [0:40] | Covers temperature values |
| scbd_humidity_cp | [0:100] | Covers humidity values |
| scbd_luminous _intensity_cp | [0:900] | Covers luminous intensity values |
| temperature_cross | scbd_enable_cp x scbd_temperature_cp | Covers temperature values when device is enabled |
| humidity_cross | scbd_enable_cp x scbd_humidity_cp | Covers humidity values when device is enabled |
| luminous_intensity _cross | scbd_enable_cp x scbd_luminous_intensity_cp | Covers luminous intensity values when device is enabled |

## Verification Scenarios and Tests

Table 17. Verification Scenarios

| Scenario | Description |
| --- | --- |
| Reset output values | Verifies that after a reset signal is applied, all DUT outputs are initialized to their expected default values. |
| Random Sensor Verification | Verifies the correct operation of the temperature, humidity, and light sensors using randomly generated input values. |
| Boundary Value Verification | Assign fixed boundary sensor values defined in the specification table. |
| Humidity Control Verification | Verifies that the humidity control logic operates correctly when humidity values remain within the normal operating range. |
| Light Intensity Control Verification | Verifies correct DUT behavior when light intensity values are within the recommended range. |
| Temperature Control Verification | Verifies the temperature control functionality when temperature values remain between 22 °C and 25 °C. |
| Enable Signal Functionality Verification | Verifies correct DUT operation when the enable signal is repeatedly toggled through multiple button presses. |
| Button Functionality Verification | Verifies the correct response of the DUT to rapid button presses and ensures proper button handling. |
| Periodic Shutdown Verification | Verifies that the environmental control module can be disabled and re-enabled during operation. |

Table 18. List of implemented tests for DUT verification

| Test file name | Test description |
| --- | --- |
| frequent_button_test | Test that DUT enables and disables correctly when button is pressed rapidly. |
| humidity_test | Test the DUT behavior for different humidity values from all operating ranges |
| luminosity_test | Test the DUT behavior for different luminosity values from all operating ranges |
| temperature_test | Test the DUT behavior for different temperature values from all operating ranges |
| sensor_limit_values_test | Test that DUT behaves correctly when sensor inputs are set to boundary values. |
| sensor_test | Test the correct operation of the environmental control system using randomly generated temperature, humidity, and light intensity values. |

## Results

Running the regression produced the following results:

- Sensor functional coverage: 100%

- Actuator functional coverage: 100%

- Button functional coverage: 100%

- Scoreboard coverage: 100%

All assertions passed without failures throughout the simulation