Department of Electronics and Computers

Study programme: Applied Electronics

Verification Project

Smart Lamp Controller

Table of Contents

1	The digital design of the circuit	4

1.1	 Block Diagram	4

1.2	Description of the general functionality of the DUT (Device Under Test)	4

1.3	DUT Interfaces	5

1.4	List of DUT Functionalities	6

1.5	Schematic Diagram	7

1.6	Waveforms	7

2	Digital Circuit Verification	9

2.1	Verification Environment Architecture	9

2.1.1	Sensor Agent	9

2.1.2	Sensor Transaction	9

2.1.3	Sensor Driver	10

2.1.4	Sensor Monitor	10

2.1.5	Sensor Coverage Collector	10

2.1.6	Sensor Sequences	10

2.1.7	Lamp Agent	10

2.1.8	Lamp Transaction	11

2.1.9	Lamp Monitor	11

2.1.10	Lamp Coverage Collector	11

2.1.11	Button Agent	11

2.1.12	Button Transaction	11

2.1.13	Button Driver	11

2.1.14	Button Monitor	11

2.1.15	Button Coverage Collector	12

2.1.16	Button Sequences	12

2.1.17	Scoreboard	12

2.1.18	Scoreboard Coverage Collector	12

2.2	Verification Checklist	13

2.2.1	Assertions	13

2.3	Functional Coverage Elements	13

2.4	Verification Scenarios and Tests	15

2.5	Results	15

# The digital design of the circuit

Block Diagram 
General Functionality Description of the DUT (Device Under Test) 
DUT Interfaces

List of DUT Functionalities

Schematic Diagram

Waveforms

Register Table

## Block Diagram

Figure 1. Block diagram of the Smart Lamp Controller

## Description of the general functionality of the DUT (Device Under Test)

The DUT controls the light intensity of a lamp. Its functionality depends on two modes: manual mode and automatic mode. The initial state of the lamp is off. A short press from this state switches directly to automatic mode, and a long press when the lamp is off, switches to manual mode. The lamp switches between the two modes by long pressing its button for minimum 20 clock cycles.

In manual mode, the light intensity changes between 4 states by short pressing the button for less than 20 clock cycles, starting with the lamp switched off. In automatic mode, the DUT changes the light intensity based on the values received from a light sensor. The DUT has 4 pre-defined ranges, representing the lamp illumination steps. The sensor data is transmitted using the valid-ready protocol. The DUT only receives the value from the sensor when the valid signal is active. When the valid is not active, the DUT keeps the previously read value.

## DUT Interfaces

Table 1. System signals generated in the testbench

| General Signals |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| clk | I | 1 | Clock signal |
| reset | I | 1 | Asynchronous reset, active low |

Table 2. Sensor Interface

| Sensor Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| brightness | I | 8 | Environmental brightness |
| valid | I | 1 | Valid signal |
| ready | O | 1 | Ready signal |

Figure 2. Valid-ready protocol

Table 3. Button Interface

| Button Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| button | I | 1 | For changing modes and states, active low |

Table 4. Lamp Interface

| Lamp Interface |  |  |  |
| --- | --- | --- | --- |
| Signal Name | I/O | Size | Description |
| lamp_control | O | 2 | Lighting thresholds |

## List of DUT Functionalities

- Reads button state

- Counts the number of clock cycles when the button is pressed

- Receives environmental brightness from light sensor in automatic mode

- Calculates lamp’s light intensity based on press length and sensor data

Table 5. Brightness ranges

| Light Intensity | Sensor Values Range | DUT Output Value |
| --- | --- | --- |
| On 2 | 0-63 | 11 |
| On 1 | 64-127 | 10 |
| On 0 | 128-191 | 01 |
| Off | 192-255 | 00 |

Figure 3. Finite State Machine describing the functionality of the smart lamp

## Schematic Diagram

Figure 4. DUT Block Diagram

Table 6. Internal signals of DUT

| Signal name | Description |
| --- | --- |
| short_push | Indicates that the button has been pressed short |
| long_push | Indicates that the has been pressed long |
| s_off | Indicates that the DUT is in the OFF state, lamp is off. |
| s_on0 | Indicates that the DUT is in ON0 state. |
| s_on1 | Indicates that the DUT is in ON1 state. |
| s_on2 | Indicates that the DUT is in ON2 state. |
| s_disable | Indicates that the DUT is turned off. |
| s_auto | Indicates that the DUT is in the Automatic mode. |

## Waveforms

Figure 5. Waveforms from the DUT in automatic mode

Figure 6. Waveforms from the DUT in manual mode

# Digital Circuit Verification

Verification Environment Architecture

Verification Checklist

Functional Coverage Elements

Verification Scenarios

Tests Results

## Verification Environment Architecture

Figure 7. Overview of the components necessary for sending stimuli to the DUT

### Sensor Agent

The Sensor Agent is an active agent that encapsulates the Sequencer, Driver and Monitor into a single entity, connecting the components with the DUT using the virtual interface.

### Sensor Transaction

Table 7. Sensor Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| sensor | rand bit | 8 | [0:255] | Value of environmental brightness |

### Sensor Driver

The Sensor Driver interacts with the DUT via the Sensor Interface, sending the transactions generated by the Sensor Sequencer. The transactions follow the valid-ready protocol.

The driver receives transactions from the sequencer, waits for the ready signal to assert, asserts valid signal and drives brightness signals, and after a clock cycle de-asserts the valid signal.

### Sensor Monitor

The Sensor Monitor waits for valid and ready signals to be both asserted, samples the brightness signal from the Sensor Virtual Interface, reconstructs the transaction and sends it to the Scoreboard and Sensor Coverage Collector.

### Sensor Coverage Collector

The Sensor Coverage Collector is instantiated inside the Sensor Monitor and receives transactions from it. The coverage collector contains sensor_cg cover group with sensor_state cover point. It has bins for every range of environmental brightness and for lower limits and upper limits of every range.

### Sensor Sequences

The Sensor Sequences generate stimulus for the Sensor Driver. Each sequence creates one or more constrained-random transactions that the driver sends to Sensor Interface. The following sequences are implemented:

Table 8. Sensor Sequences

| Sequence | Description |
| --- | --- |
| balanced_data _sequence | Sends values of every environmental brightness range equally. |
| sensor_limit_values _sequence | Sends only lower and upper limits of every environmental brightness range. |
| sensor_sequence | Sends random values of environmental brightness. |

### Lamp Agent

The Lamp Agent is a passive agent that contains only the Monitor and connects it with the DUT using the virtual interface. The Lamp Interface has only output signals from DUT, meaning no driver or sequencer is needed.

### Lamp Transaction

Table 9. Lamp Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| light_level_out | rand lamp_state | 2 | {LIGHT_OFF, ON_LEVEL_0, ON_LEVEL_1, ON_LEVEL_2} | Lamp’s light intensity |

### Lamp Monitor

The Lamp Monitor samples the light level signal every time its value changes, reconstructs the transaction and sends it to the Scoreboard and Lamp Coverage Collector.

### Lamp Coverage Collector

The Lamp Coverage Collector is instantiated inside the Lamp Monitor and receives transactions from it. The coverage collector contains lamp_state_cg cover group with lamp_state cover point. It has bins for every light level of the lamp.

### Button Agent

The Button Agent is an active agent that encapsulates the Sequencer, Driver and Monitor into a single entity, connecting the components with the DUT using the virtual interface.

### Button Transaction

Table 10. Button Transaction

| Field Name | Type | Size | Values | Description |
| --- | --- | --- | --- | --- |
| button | rand button_state | 2 | {UNPUSHED_BUTTON, SHORT_PUSH_BUTTON, LONG_PUSH_BUTTON} | Type of button press |

### Button Driver

The Button Driver receives transactions from the Button Sequencer, de-asserts the button signal, waits for several clock cycles based on the press type and asserts the button signal.

### Button Monitor

The Button Monitor waits for the button signal to de-assert, counts the number of clock cycles during which it stays low, decides the type of button press, reconstructs the transaction and sends it to the Scoreboard and Button Coverage Collector.

### Button Coverage Collector

The Button Coverage Collector is instantiated inside the Button Monitor and receives transactions from it. The coverage collector contains button_cg cover group with button_state cover point. It has bins for every type of button press.

### Button Sequences

The Button Sequences generate stimulus for the Button Driver. Each sequence creates one or more constrained-random transactions that the driver sends to Button Interface. The following sequences are implemented:

Table 11. Button Sequences

| Sequence | Description |
| --- | --- |
| auto_mode_sequence | Button is only shortly pressed to enable and disable automatic mode. |
| manual_mode _sequence | First push button is long to enable manual mode. The rest are short to switch between lamp light intensities. |
| mode_switch _sequence | Button is only long pressed to switch between automatic mode and manual mode. |
| one_long_push_button_sequence | Button is pressed long one time. Used after manual_mode_sequence in case Sensor Driver has remaining transactions. |
| random_sequence | Random type of button press. |

### Scoreboard

The Scoreboard receives transactions from all monitors.

When receiving a Button Transaction, the scoreboard predicts DUT’s operating mode and the next lamp light level if DUT is in manual mode.

When receiving a Sensor Transaction, the scoreboard uses it to predict the next Lamp Transaction when DUT is in automatic mode.

All predicted Lamp Transactions are stored in a queue.

When receiving a Lamp Transaction, the scoreboard compares it with the predicted transaction, generating an error message if the checker fails.

### Scoreboard Coverage Collector

The Scoreboard Coverage Collector is instantiated inside the Scoreboard and receives transactions from it. The coverage collector contains lamp_transition_coverage_cg cover group with 4 cover points: next_lamp_state, current_lamp_state, current_mode, next_mode; and 3 crosses: lamp_transition, operation_transition, lamp_operation_transition.

## Verification Checklist

### Assertions

All Sensor assertions are disabled when reset is active to prevent false assertion failures.

Table 12. Sensor Interface Assertions

| Assertion | Description |
| --- | --- |
| valid_ready | Verifies that valid signal is asserted only when ready signal is asserted. |
| valid_puls | Verifies that valid signal is a single pulse. |

## Functional Coverage Elements

Table 13. Sensor cover group

| sensor_cg |  |  |  |
| --- | --- | --- | --- |
| Cover point | Bins | Values | Description |
| sensor_state | off | [193:254] | Lamp light off |
|  | on0 | [127:190] | Lamp light in ON0 state |
|  | on1 | [65:126] | Lamp light in ON1 state |
|  | on2 | [1:62] | Lamp light in ON2 state |
|  | limit_value_0 | 0 | Lower limit for light in ON2 state |
|  | limit_value_1 | 63 | Upper limit for light in ON2 state |
|  | limit_value_2 | 64 | Lower limit for light in ON1 state |
|  | limit_value_3 | 127 | Upper limit for light in ON1 state |
|  | limit_value_4 | 128 | Lower limit for light in ON0 state |
|  | limit_value_5 | 191 | Upper limit for light in ON0 state |
|  | limit_value_6 | 192 | Lower limit for light off |
|  | limit_value_7 | 255 | Upper limit for light off |

Table 14. Lamp cover group

| lamp_state_cg |  |  |  |
| --- | --- | --- | --- |
| Cover point | Bins | Values | Description |
| lamp_state | LIGHT_OFF | LIGHT_OFF | Lamp light off |
|  | ON_LEVEL_0 | ON_LEVEL_0 | Lamp light in ON0 state |
|  | ON_LEVEL_1 | ON_LEVEL_1 | Lamp light in ON1 state |
|  | ON_LEVEL_2 | ON_LEVEL_2 | Lamp light in ON2 state |

Table 15. Button cover group

| button_cg |  |  |  |
| --- | --- | --- | --- |
| Cover point | Bins | Values | Description |
| button_state | UNPUSHED_BUTTON | UNPUSHED_BUTTON | Button unpushed |
|  | SHORT_PUSH_BUTTON | SHORT_PUSH_BUTTON | Button short pressed |
|  | LONG_PUSH_BUTTON | LONG_PUSH_BUTTON | Button long pressed |

Table 16. Scoreboard cover group

| lamp_transition_coverage_cg |  |
| --- | --- |
| Cover point | Description |
| next_lamp_state | Covers the next predicted light intensity |
| current_lamp_state | Covers the current light intensity |
| next_mode | Covers the next predicted operation mode |
| current_mode | Covers the current operation mode |
| lamp_transition | Covers the transition between light intensities |
| operation_transition | Covers the transition between operation modes |
| lamp_operation_transition | Covers the transition between light intensities in both operation modes |

## Verification Scenarios and Tests

Table 17. Verification Scenarios

| Scenario | Description |
| --- | --- |
| Sensor Interval Verification | Random values corresponding to each interval are applied while the DUT operates in automatic mode. |
| Boundary Value Verification | Assign fixed boundary sensor values defined in the specification table. |
| Continuous Button Press Verification | Button is pressed continuously for long durations to switch between operation modes. |
| Short and Long Button Press Verification | Use representative clock cycle values for short and long presses. |

Table 18. List of implemented tests for DUT verification

| Test file name | Test description |
| --- | --- |
| auto_mode_test | Test that DUT switches correctly to automatic mode. |
| manual_mode_test | Test that DUT switched between lamp light levels correctly in manual mode. |
| long_push_button _test | Test that DUT switches between operational modes correctly. |
| sensor_limit_values_test | Test that DUT sets the lamp light level correctly when sensor sends boundary values. |
| sensor_test | Test that DUT sets the lamp light level correctly based on the environment brightness received from the sensor in automatic mode. |
| global_test | Test that DUT behaves correctly under randomly generated scenarios. |

## Results

Running the regression produced the following results:

- Sensor functional coverage: 100%

- Lamp functional coverage: 100%

- Button functional coverage: 100%

- Scoreboard coverage: 100%

Sensor assertions passed without failures throughout the simulation